Faculty of Engineering and IT
School of Mechanical and Mechatronic Engineering
41069 Robotics Studio 2

# Social Tour Guide Robot

Client:

Coach:

Team Name:  T4

Team Members: 4

| Full Name | Student ID | Expertise |
| --- | --- | --- |
| Karan Sharma | 26276485 | Navigation Behaviour & Route Intelligence |
| Vinh Anh Nguyen | 25629813 | Path planning and Control |
| Darren Padamittam Jacob | 25755416 | Electronics and Hardware Implementation |
| Hayden Law | 24957767 | Functions-Actions interface |

# Overview

As organizations increasingly seek to automate repetitive and information-intensive tasks, there is growing demand for solutions that can deliver accurate, real-time information efficiently and cost-effectively. In environments such as universities, corporate offices, exhibitions, and public facilities, guiding visitors and communicating relevant information remains a time-consuming responsibility for staff.

This project proposes the development of a socially interactive tour guide robot capable of autonomously guiding visitors through an environment while delivering contextual information. The robot will enhance customer experience by combining navigation capabilities with natural human–robot interaction.

The system will be designed to:

- Greet visitors and initiate guided tours

- Navigate safely through dynamic, crowded environments

- Provide explanations and contextual information at designated locations

- Respond to voice commands and basic conversational prompts

- Use gestures and expressive behaviors to create a welcoming interaction

A key objective of the project is to ensure that the robot operates safely and comfortably in socially dynamic spaces. The robot must move in a manner that respects personal space, adapts to pedestrian flow, and remains approachable and intuitive for users.

Rather than relying solely on pre-programmed routes, the system will support adaptable route management, allowing tours to be modified or expanded based on operational needs. The robot should also maintain persistent knowledge of its environment to improve performance over time.

The ultimate goal is to deliver a reliable, interactive, and user-friendly tour guide system that enhances visitor engagement while reducing operational workload for staff.

In pursuit of the aforementioned goals, there are some expected challenges pertinent to both design and implementation. First, as our alpha development phase is done using a turtlebot, there is the immense challenge of moving the code base from this robot to a Pepper robot on the next phase. Furthermore, as some intended functionalities of the robot, like socially aware path planning, visual SLAM, and human-robot interactions, can be considered advanced research topics, implementing and integrating them can prove a complicated venture.

# System Evaluation

Describe in detail how your fully integrated system should operate. You should write from the user (client) perspective, describing the key features, functionalities and behaviours. We will use these criteria at the Project Handover to evaluate your system.

Note: you will have a chance after week 7 to modify the D, HD and Perfect criteria.

| System Evaluation |  |  |
| --- | --- | --- |
| P (MVP) | The robot is able to make a map with human control, localize itself within the given map, and path planning without collision as well as travel according to the path. The robot is able to move around this map whilst avoiding collisions. The robot is able to receive basic destinations and questions from a human through a basic graphical interface and perform these tasks as well as give pre-programmed answers. |
| C | The robot is able to automatically perform a programmed tour guide routine by moving around the environment and providing information at the right timing. The robot can move around in an unknown environment whilst avoiding collision. |
| D | The robot can add new routes, waypoints, and artifact information to its current map through an easy manual process and then automatically give a tour following this new route. |
| HD | The robot can perform high-level actions like traveling, providing information, route selection, or switching modes through speech input. Given a list of random artifacts/waypoints (a subset of the total number), an optimal route will be made to visit all of these waypoints. The robot is capable of returning to a default position and orientation (docking at a station) when a tour finishes or when battery is low. |
| Perfect | The robot is able to receive input from humans in the form of speech and convert it into high-level actions like traveling, pointing, or body language articulation. The robot is able to process information from camera input in combination with speech (what is this object? (as human points to an object)) Socially aware navigation by adjusting the speed according to the number of humans in the environment. The robot can follow a specific person around and stop following after a trigger (like help me, etc.). When the person stops, the robot also stops. The tour guide robot can detect emotions of tourists and integrate it into responses. |

# Subsystem Evaluation

Describe in detail how individual subsystems should operate and be evaluated. Ideally, a subsystem should be able to function independently from other subsystems, with clearly defined inputs and outputs. Although you are free to define the project however you like, as this is a robotics project, your subsystems need to follow these themes/topics:

| Subsystem 1 – Locomotion of social guide robot |  | Lead by: Vinh Anh Nguyen |  |
| --- | --- | --- | --- |
| P | The robot is able to make a map with human control, localize itself within the given map, and path planning without collision as well as travel according to the path. |  |
| C | Interface with higher level behavior layer, successfully integrating camera, speaker, and external RGBD camera. SLAM implementation for turtlebot. The robot can navigate through an arbitrary environment without collisions. |  |
| D | Implement automatic route creation. The human will manually control the robot to create a route and mark points on that route as waypoints. The robot will then follow this route and give information at the waypoints. |  |
| HD | Traveling salesman tour route making using the turtlebot. Given a list of random artifacts/waypoints (a subset of the total number), an optimal route will be made to visit all of these whilst minimizing a certain cost function. |  |
| Perfect | The robot can follow a specific person around and stop following after a trigger (like help me, etc.). When the person stops, the robot also stops. Automatically docking and charging. When battery is low, or when a tour terminates, the robot will return automatically to its station in a specified pose (maybe even back into a charging port). |  |
| Subsystem 2 – Gestures and Communication interface |  | Lead by: Hayden Law |  |
| P | Pepper Robot can perform basic limb gestures using its hands and head in response to stimuli. Pepper Robot will perform body language to mimic human body language. This includes handwaving while speaking. Pepper Robot can speak explanations of the current exhibition it is focused on. Speech will be based on the programmed response inherent to the current exhibit Pepper Robot is focused on. |  |
| C | Pepper Robot is able to use the screen to supplement explanations. An image of the exhibit will be shown during explanations, and the words used in speech will be shown as scrolling text. |  |
| D | Pepper Robot will have responses to whichever state it may be in, i.e nodding during listening and displaying symbols on screen for locomotion, speaking, listening, idle etc.. Generally, Pepper Robot will be dynamic and moving in any state other than idle. |  |
| HD | Guests will be able to ask Pepper Robot to change the size of text, so that they may have an easier time reading it. This will be facilitated by Speech to Text capabilities and audio capturing. |  |
| Perfect | Pepper Robot’s screen will not only display images, but also highlight certain aspects of the image based on what about the exhibit it is talking about. This can be done by zooming in, or circling around it. Pepper Robot will respond dynamically using outputs from an LLM. This would allow for customised answers to questions about any exhibit. Pepper Robot will display an arrow on screen and point to where its next destination is on the tour. |  |
| Subsystem 3 - Human-machine interface and behavior |  | Lead by: Karan Sharma |  |
| P | - Speech-to-text integration with predefined command recognition. - Basic command-to-action mapping (e.g., Follow, Stop, Navigate). - Ability to trigger navigation goals via ROS2. - Simple verbal responses to recognized commands. - Basic Behaviour Tree structure controlling navigation start/stop. |  |
| C | - Named route saving and retrieval through speech commands. - Demonstration mode controller (start/stop route recording). - Structured tour sequence execution. - Ability to pause and resume navigation upon interruption. - Context-aware responses during tour. - Basic state tracking of current task (e.g., touring, following, idle). |  |
| D | - Full Behaviour Tree orchestration managing: - Follow mode - Route saving - Route replay - Tour explanation - Route replay intelligence (waypoint sequencing & refinement trigger). - Interruption handling with state recovery (pause → respond → resume). - Dialogue state memory during tour execution. - Gesture-speech synchronization. - Social navigation policy control (speed modulation, facing user, stopping at waypoints). |  |
| HD | - Modular Behaviour Tree architecture with scalable action primitives: - NavigateTo(Location) - FollowUser() - SaveRoute(Label) - ReplayRoute(Label) - Explain(Location) - Intelligent transition between operational modes without system reset. - Robust failure handling (e.g., lost localization → recovery routine). - Dynamic route management integrated with navigation subsystem. - Socially-aware movement decisions during explanation (stop, orient, gesture). - Seamless integration of speech, navigation, and embodiment into a unified control framework. |  |
| Perfect | At the Perfect level, the robot should be able to engage in natural conversation with users and convert spoken commands into structured tokens that can be interpreted by the behaviour tree controller. These tokens represent the user’s intent and allow the controller to make decisions based on the requested task. The behaviour tree will then classify the task into operational modules such as route planning, navigation, and gesture or interaction control. Each module will execute the required actions, allowing the robot to translate high-level conversational commands into coordinated movements and interactions within the environment. |  |
| Subsystem 4 - Integrate ROS code with Pepper |  | Lead by: Darren Padamittam Jacob |  |
| P | - Able to operate and work on pepper from computer and ROS - Pepper Robot hand and body will be programmed to point, emote, gesture, and generally use its hands to supplement speech and general explanation. |  |
| C | Basic functionality achieved in the TurtleBot is implemented in Pepper. - Pepper can do localization, navigation and Path planning. - Interface with higher level behaviour layer, successfully integrating camera, speaker, and RGBD camera. |  |
| D | Human machine Interface is achieved in pepper - Emergency stop functionality triggered by unsafe proximity or unexpected obstacles - Redundant failsafe mechanisms to handle sensor failure, communication drops, or localization uncertainty - A layered safety net architecture to mitigate risks across multiple operational scenarios |  |
| HD | - the robot shall demonstrate fully autonomous navigation within a pre-built environmental map using its onboard RGB-D camera and lidar for localization and depth perception. |  |
| Perfect | - Pepper can navigate an environment without collisions. - It can communicate with the people using gestures - Real-time human detection and proximity monitoring - Dynamic speed adjustment based on human distance |  |

References

[1] Core Challenges of Social Robot Navigation: A Survey

[2] Survey of Emotions in Human–Robot Interactions: Perspectives from Robotic Psychology on 20 Years of Research

[3] Humanoid Robots in Tourism and Hospitality—Exploring Managerial, Ethical, and Societal Challenges